| EMBEDDED SYSTEMS · CASE STUDY | Dikshant Agrawal |
| --- | --- |

Reaction Game on Erika RTOS

Real-time human reaction measurement running on OSEK task scheduler

| Role Sole Developer | Period M.Sc. coursework | Stack C++, Erika RTOS, OSEK, PWM, GPIO |
| --- | --- | --- |

OVERVIEW

A bare-metal reaction-speed game implemented on an embedded target running the Erika (OSEK) real-time operating system. The firmware randomly triggers LED patterns with PWM fade effects using scheduled RTOS tasks; the player presses a button and the system captures the precise reaction time in milliseconds and displays it on a 7-segment display or serial terminal.

WHAT WAS BUILT

Configured Erika OSEK task set: two periodic tasks (LED driver, debounce) and one event-triggered task (measurement)

PWM fade effect on LEDs using hardware timer peripheral — smooth brightness ramp controlled from RTOS task

High-resolution reaction time capture using hardware timer overflow interrupt, accurate to ±1 ms

Results logged over UART for PC-side analysis and personal best tracking

Clean C++ class design separating hardware abstraction, game state machine, and display logic

KEY OUTCOME

Demonstrated RTOS task scheduling, hardware timer usage, and clean C++ OOP on bare-metal. Reaction measurements were consistent within ±1 ms, validating the real-time accuracy of the Erika scheduler implementation.

agrawaldikshant3001@gmail.com  ·  Darmstadt, DE  ·  M.Sc. Embedded Systems, h_da