| EMBEDDED SYSTEMS · CASE STUDY | Dikshant Agrawal |
| --- | --- |

Timing Analyzer

8-channel digital timing analyzer with microsecond resolution over UART

| Role Sole Developer | Period Personal / coursework | Stack C++, GPIO, Hardware Timer, UART, Interrupt, STL |
| --- | --- | --- |

OVERVIEW

A software timing analyzer implemented on an embedded MCU that captures digital signal transitions across 8 input channels with microsecond resolution. The tool is intended for embedded debugging — allowing developers to visualize timing relationships between signals such as SPI chip select, data lines, and interrupt triggers. Results are streamed over UART to a PC terminal in a human-readable channel-view format.

WHAT WAS BUILT

Hardware timer configured for 1 MHz tick rate providing 1 µs resolution; timer captures stored in ISR

GPIO interrupt handler for each channel records (channel_id, timestamp, level) into a ring buffer

C++ class ChannelRecorder encapsulates per-channel capture logic; TimingView formats output

Post-capture analysis calculates pulse widths, periods, and delta-t between any two channels

UART streaming at 115200 baud outputs annotated waveform ASCII art for immediate inspection

Configurable trigger: start capture on rising/falling edge of any selected channel

KEY OUTCOME

Successfully measured timing relationships between SPI clock, MOSI, and CS signals with sub-2 µs jitter, providing a lightweight, no-oscilloscope debugging workflow useful during protocol bring-up on resource-constrained targets.

agrawaldikshant3001@gmail.com  ·  Darmstadt, DE  ·  M.Sc. Embedded Systems, h_da