| EMBEDDED SYSTEMS · CASE STUDY | Dikshant Agrawal |
| --- | --- |

4th-Order Bandpass Amplifier PCB

Schematic capture, 4-layer PCB layout and breadboard prototype — SDHD Lab, h_da SS25

| Role Sole Designer | Period SS25 · M.Sc. h_da | Stack KiCad, OPA340, THT, 4-layer PCB, FreeSoc2 |
| --- | --- | --- |

OVERVIEW

A complete hardware design exercise for the System Driven Hardware Design (SDHD) lab at Hochschule Darmstadt (SS25, Prof. Dr.-Ing. Stephan Bannwarth). The task covered the full hardware design flow: hand-calculation of component values, schematic capture, PCB layout under tight manufacturing constraints, and breadboard prototype verification. The circuit is a 4th-order active bandpass amplifier built from two cascaded OPA340 op-amp stages (Stage 1: A=100 with R4=220k; Stage 2: A=3 with R6=100k), powered from a FreeSoc2 board.

WHAT WAS BUILT

Hand-calculated R3 and R5 values and corner frequencies for each Sallen-Key stage; verified overall 3 dB bandwidth

Schematic captured in KiCad: OPA340 in DIL8 socket, E24-series THT resistors, decoupling capacitors (C2=22µF, C3=1.5nF, C5=4.7µF, C6=3.3nF), power indicator LED

PCB layout on Arduino UNO Shield template (KiCad): 4-layer stackup (Signal – GND – Power – Signal), analog and digital ground planes separated

Design rule compliance: min track width 0.20 mm, PTH vias only (0.8 mm / 0.4 mm drill), no buried or blind vias, components within Arduino shield market area

Digital part: 3-pin 90° connector for radar sensor interface, push-button, RGB status LEDs (red / yellow / green)

PCB specification submitted to Elekonta for fabrication; breadboard prototype built and signal verified on oscilloscope

KEY OUTCOME

DRC passed with zero errors. Breadboard prototype confirmed correct bandpass response: Stage 1 high-pass and Stage 2 low-pass corner frequencies combined to achieve the specified 4th-order characteristic. The design demonstrated the full hardware design workflow from hand calculation through verified PCB layout — a key skill for embedded hardware roles in Germany.

agrawaldikshant3001@gmail.com  ·  Darmstadt, DE  ·  M.Sc. Embedded Systems, h_da